PACKET TRACER LAB GUIDE

Configure Numbered Standard IPv4 ACLs

| TARGET DEVICES Routers R1, R2, R3 (Cisco IOS) | ACCESS CONTROL TYPE Numbered Standard IPv4 ACL (List #1) |
| --- | --- |
| ROUTING PROTOCOL EIGRP (Pre-configured full mesh) | PLACEMENT RULE Closest to Destination on Outbound Interface (G0/0) |

📌 LAB SCENARIO & OVERVIEW: Standard access control lists (ACLs) are router configuration scripts that filter packets based solely on source IP address. In this activity, you will plan, configure, and verify numbered standard ACLs (using ACL number 1) on R2 and R3 to satisfy two distinct organizational traffic security policies while permitting all other authorized communication across the EIGRP network.

📌 POLICY IMPLEMENTATION & PLACEMENT CRITERIA: • Policy 1 (Router R2): Block network 192.168.11.0/24 (PC2) from accessing WebServer (192.168.20.254 on 192.168.20.0/24). Permit all other traffic. • Policy 2 (Router R3): Block network 192.168.10.0/24 (PC1) from communicating with 192.168.30.0/24 (PC3). Permit all other traffic. • Placement Strategy: Since standard ACLs filter only by source IP, they MUST be applied as close to the destination as possible (outbound on G0/0 of R2 and R3) to prevent inadvertently blocking traffic to other subnets.

Network Addressing Table

| Device | Interface | IP Address | Subnet Mask | Default Gateway | Connected Role / Function |
| --- | --- | --- | --- | --- | --- |
| R1 | G0/0 | 192.168.10.1 | 255.255.255.0 | N/A | LAN Gateway for PC1 Network |
| R1 | G0/1 | 192.168.11.1 | 255.255.255.0 | N/A | LAN Gateway for PC2 Network |
| R1 | S0/0/0 | 10.1.1.1 | 255.255.255.252 | N/A | WAN Link to R2 (10.1.1.0/30) |
| R1 | S0/0/1 | 10.3.3.1 | 255.255.255.252 | N/A | WAN Link to R3 (10.3.3.0/30) |
| R2 | G0/0 | 192.168.20.1 | 255.255.255.0 | N/A | LAN Gateway for WebServer (ACL 1 Outbound) |
| R2 | S0/0/0 | 10.1.1.2 | 255.255.255.252 | N/A | WAN Link to R1 (10.1.1.0/30) |
| R2 | S0/0/1 | 10.2.2.1 | 255.255.255.252 | N/A | WAN Link to R3 (10.2.2.0/30) |
| R3 | G0/0 | 192.168.30.1 | 255.255.255.0 | N/A | LAN Gateway for PC3 (ACL 1 Outbound) |
| R3 | S0/0/0 | 10.3.3.2 | 255.255.255.252 | N/A | WAN Link to R1 (10.3.3.0/30) |
| R3 | S0/0/1 | 10.2.2.2 | 255.255.255.252 | N/A | WAN Link to R2 (10.2.2.0/30) |
| PC1 | NIC | 192.168.10.10 | 255.255.255.0 | 192.168.10.1 | LAN Host on R1 G0/0 |
| PC2 | NIC | 192.168.11.10 | 255.255.255.0 | 192.168.11.1 | LAN Host on R1 G0/1 |
| PC3 | NIC | 192.168.30.10 | 255.255.255.0 | 192.168.30.1 | LAN Host on R3 G0/0 |
| WebServer | NIC | 192.168.20.254 | 255.255.255.0 | 192.168.20.1 | Enterprise WebServer on R2 G0/0 |

Part 1: Plan an ACL Implementation

Step 1: Investigate Baseline Network Reachability

• Before configuring any access lists, verify that full mesh IP reachability exists across the network.

• Open Command Prompt on PC1 and ping all endpoints: PC2 (192.168.11.10), PC3 (192.168.30.10), and WebServer (192.168.20.254).

• Confirm that EIGRP routing tables are populated on R1, R2, and R3, and all baseline pings succeed with 100% success rate.

PC1> ping 192.168.11.10 # Ping PC2 - Success (4/4) PC1> ping 192.168.20.254 # Ping WebServer - Success (4/4) PC1> ping 192.168.30.10 # Ping PC3 - Success (4/4)

Step 2: Evaluate Policy Requirements & Determine ACL Placements

• Policy A (Router R2 Policy):

•   - Requirement: The 192.168.11.0/24 subnet (PC2) is denied access to the WebServer (192.168.20.254 on 192.168.20.0/24).

•   - All other traffic to WebServer and between networks must be permitted.

•   - ACL Placement: Standard ACLs cannot filter by destination IP. If placed on R1, PC2 would be blocked from reaching R2 and R3 entirely! Therefore, ACL 1 must be placed on R2 outbound on interface GigabitEthernet 0/0.

• Policy B (Router R3 Policy):

•   - Requirement: The 192.168.10.0/24 subnet (PC1) is denied access to communicate with 192.168.30.0/24 (PC3).

•   - All other traffic to PC3 and between networks must be permitted.

•   - ACL Placement: Placed on R3 outbound on interface GigabitEthernet 0/0 directly protecting the PC3 LAN.

📌 TECHNICAL DEEP DIVE: WILDCARD MASK CALCULATION: Wildcard masks are calculated by subtracting the subnet mask from 255.255.255.255: 255.255.255.255 - 255.255.255.0 (/24) = 0.0.0.255 In binary, a '0' bit means 'must match exactly', while a '1' bit means 'ignore this bit'. Therefore, '192.168.11.0 0.0.0.255' matches any host whose first three octets are 192.168.11.

Part 2: Configure, Apply, and Verify Numbered Standard ACLs

Step 1: Configure and Apply Numbered Standard ACL 1 on Router R2

• Access Router R2 CLI and enter Global Configuration mode (configure terminal).

• Create Standard ACL 1 to deny the 192.168.11.0/24 network: access-list 1 deny 192.168.11.0 0.0.0.255

• Append an explicit permit statement to allow all other traffic: access-list 1 permit any

• Review ACL statements using show access-lists.

• Apply ACL 1 outbound on interface GigabitEthernet 0/0: ip access-group 1 out.

R2> enable R2# configure terminal R2(config)# access-list 1 deny 192.168.11.0 0.0.0.255 R2(config)# access-list 1 permit any R2(config)# exit R2# show access-lists Standard IP access list 1 10 deny 192.168.11.0, wildcard bits 0.0.0.255 20 permit any R2# configure terminal R2(config)# interface GigabitEthernet0/0 R2(config-if)# ip access-group 1 out R2(config-if)# end

Step 2: Configure and Apply Numbered Standard ACL 1 on Router R3

• Access Router R3 CLI and enter Global Configuration mode.

• Create Standard ACL 1 to deny the 192.168.10.0/24 network: access-list 1 deny 192.168.10.0 0.0.0.255

• Append the permit statement for remaining traffic: access-list 1 permit any

• Review ACL statements using show access-lists.

• Apply ACL 1 outbound on interface GigabitEthernet 0/0: ip access-group 1 out.

R3> enable R3# configure terminal R3(config)# access-list 1 deny 192.168.10.0 0.0.0.255 R3(config)# access-list 1 permit any R3(config)# exit R3# show access-lists Standard IP access list 1 10 deny 192.168.10.0, wildcard bits 0.0.0.255 20 permit any R3# configure terminal R3(config)# interface GigabitEthernet0/0 R3(config-if)# ip access-group 1 out R3(config-if)# end

📌 NUMBERED ACL RANGE CONVENTION: Standard ACL numbers range from 1 to 99 and 1300 to 1999. In Cisco IOS, access-list numbers are locally significant to each router. Using ACL 1 on both R2 and R3 is completely independent and valid.

Step 3: Verify Interface Bindings & Placement

• Verify outbound ACL placement on R2 and R3 using show ip interface gigabitethernet 0/0 or show running-config.

R2# show ip interface gigabitethernet 0/0 GigabitEthernet0/0 is up, line protocol is up Internet address is 192.168.20.1/24 Outgoing access list is 1 # <-- Outbound active Inbound access list is not set R3# show ip interface gigabitethernet 0/0 GigabitEthernet0/0 is up, line protocol is up Internet address is 192.168.30.1/24 Outgoing access list is 1 # <-- Outbound active Inbound access list is not set

Step 4: Execute Functional Verification Tests (Ping Matrix)

• Test 1: Ping from PC1 (192.168.10.10) to PC2 (192.168.11.10) -> Must SUCCEED (Local R1 routing).

• Test 2: Ping from PC1 (192.168.10.10) to WebServer (192.168.20.254) -> Must SUCCEED (Permitted by R2 ACL 1 rule 20).

• Test 3: Ping from PC2 (192.168.11.10) to WebServer (192.168.20.254) -> Must FAIL (Blocked by R2 ACL 1 rule 10).

• Test 4: Ping from PC1 (192.168.10.10) to PC3 (192.168.30.10) -> Must FAIL (Blocked by R3 ACL 1 rule 10).

• Test 5: Ping from PC2 (192.168.11.10) to PC3 (192.168.30.10) -> Must SUCCEED (Permitted by R3 ACL 1 rule 20).

• Test 6: Ping from PC3 (192.168.30.10) to WebServer (192.168.20.254) -> Must SUCCEED (Permitted by R2 ACL 1 rule 20).

# PC1 (192.168.10.10) Tests: PC1> ping 192.168.11.10 # Success rate is 100% (4/4) -> ALLOWED PC1> ping 192.168.20.254 # Success rate is 100% (4/4) -> ALLOWED PC1> ping 192.168.30.10 # Destination host unreachable -> DENIED by R3 ACL 1 # PC2 (192.168.11.10) Tests: PC2> ping 192.168.20.254 # Destination host unreachable -> DENIED by R2 ACL 1 PC2> ping 192.168.30.10 # Success rate is 100% (4/4) -> ALLOWED # PC3 (192.168.30.10) Tests: PC3> ping 192.168.20.254 # Success rate is 100% (4/4) -> ALLOWED

Step 5: Inspect Match Counters on Routers R2 and R3

• Execute show access-lists on R2 and R3 to verify packet hit counters on the deny and permit statements.

R2# show access-lists Standard IP access list 1 10 deny 192.168.11.0, wildcard bits 0.0.0.255 (4 match(es)) 20 permit any (8 match(es)) R3# show access-lists Standard IP access list 1 10 deny 192.168.10.0, wildcard bits 0.0.0.255 (4 match(es)) 20 permit any (8 match(es))

Step 6: Check Results in Packet Tracer

• Click the Check Results button at the bottom of the Packet Tracer lab window.

• Verify that completion shows 100% and all assessment criteria are marked correct.

Policy Verification Results Matrix

| Source Host / Subnet | Target Destination | Expected Result | Enforcing Router & Rule |
| --- | --- | --- | --- |
| PC1 (192.168.10.10) | PC2 (192.168.11.10) | ✅ SUCCEEDS | R1 Local Routing (No ACL) |
| PC1 (192.168.10.10) | WebServer (192.168.20.254) | ✅ SUCCEEDS | R2 ACL 1 (Rule 20: permit any) |
| PC2 (192.168.11.10) | WebServer (192.168.20.254) | ❌ FAILS (DENIED) | R2 ACL 1 (Rule 10: deny 192.168.11.0) |
| PC1 (192.168.10.10) | PC3 (192.168.30.10) | ❌ FAILS (DENIED) | R3 ACL 1 (Rule 10: deny 192.168.10.0) |
| PC2 (192.168.11.10) | PC3 (192.168.30.10) | ✅ SUCCEEDS | R3 ACL 1 (Rule 20: permit any) |
| PC3 (192.168.30.10) | WebServer (192.168.20.254) | ✅ SUCCEEDS | R2 ACL 1 (Rule 20: permit any) |

📌 LAB COMPLETION SUMMARY: Lab completed successfully. You planned standard ACL placements nearest to the destinations, configured numbered ACL 1 on both R2 and R3 using wildcard masks, applied them outbound to G0/0, and verified traffic filtering via ICMP tests and match counters.